# Range-38’ Power Trimaran

Honda 15 hp

Full load, both engines at top speed (5,800rpm).

Fuel consumption is 1.412 gph. per engine

Speed is 12.3 knots.

100 gallons fuel gives 435 nautical miles

Half load, both engines at mid speed (4,000rpm).

Fuel consumption is 0.62 gph. per engine

Speed is 11 knots.

100 gallons fuel gives 887 nautical miles

Half load, one engine at low mid speed (3,000rpm).

Fuel consumption is 0.407 gph. per engine

Speed is 8 knots.

100 gallons fuel gives 1,965 nautical miles

Minimal load, one engine at low speed (2,000rpm).

Fuel consumption is 0.169 gph. per engine

Speed is estimated at 5 knots.

100 gallons fuel gives 2,958 nautical miles